/***************  LIBRARIES  ***************/

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#include <LightProximityAndGesture.h>   // APDS9960 (proximity/gesture)

#include <Adafruit_BMP085_U.h>          // BMP180 unified

#include <DHT.h>                        // DHT11

#include <Arduino.h>

#include <SPI.h>

#include <SD.h>

#include <FS.h>

#include <AudioFileSourceSD.h>

#include <AudioGeneratorWAV.h>

#include <AudioOutputI2S.h>

#include "driver/adc.h"

/***************  OLED CONFIG  ***************/

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

/***************  SENSORS ***************/

LightProximityAndGesture apds;

Adafruit_BMP085_Unified bmp = Adafruit_BMP085_Unified(10085);

DHT dht(4, DHT11);  // GPIO4 for DHT11

/***************  PINS ***************/

#define SOUND_PIN 32     // Clap detection sensor

#define BUZZER_PIN 33    // Buzzer pin

#define MQ135_PIN 34     // MQ135 air quality (ADC1_CH6)

#define SD_CS   5

#define SD_SCK  18

#define SD_MISO 19

#define SD_MOSI 23

/***************  AUDIO ***************/

AudioGeneratorWAV *wav;

AudioFileSourceSD *file;

AudioOutputI2S *out;

/***************  FLAGS ***************/

bool apdsAvailable = false;

/***************  HELPER: LIST SD FILES  ***************/

void listDir(fs::FS &fs, const char *dirname, uint8_t levels) {

File root = fs.open(dirname);

if (!root) {

Serial.println("Failed to open directory");

return;

}

if (!root.isDirectory()) {

Serial.println("Not a directory");

return;

}

File file = root.openNextFile();

while (file) {

if (file.isDirectory()) {

Serial.print("DIR : ");

Serial.println(file.name());

if (levels) {

listDir(fs, file.name(), levels - 1);

}

} else {

Serial.print("FILE: ");

Serial.print(file.name());

Serial.print("\tSIZE: ");

Serial.println(file.size());

}

file = root.openNextFile();

}

}

/***************  SETUP  ***************/

void setup() {

Serial.begin(115200);

Wire.begin(21,22);

pinMode(SOUND_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(BUZZER_PIN, LOW);

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED not found!");

while (true);

}

display.clearDisplay();

display.setTextColor(SSD1306_WHITE);

display.setTextSize(2);

display.setCursor(10, 20);

display.println("EchoMirror");

display.setTextSize(1);

display.setCursor(20, 45);

display.println("Waiting for clap...");

display.display();

if (apds.begin()) {

apdsAvailable = true;

apds.enableAmbientLightSensor();

apds.enableProximitySensor();

} else {

Serial.println("APDS9960 not found!");

}

if (!bmp.begin()) {

Serial.println("BMP180 not found!");

}

dht.begin();

// ADC NG driver setup for MQ135 (GPIO34)

adc1_config_width(ADC_WIDTH_BIT_12);

adc1_config_channel_atten(ADC1_CHANNEL_6, ADC_ATTEN_DB_11);

// ---- SD card init ----

SPI.begin(SD_SCK, SD_MISO, SD_MOSI, SD_CS);

if (!SD.begin(SD_CS, SPI, 4000000U)) {

Serial.println("SD card init failed! Check wiring or card.");

while(1);  // Stop here if no SD

} else {

Serial.println("SD card initialized successfully!");

uint64_t cardSize = SD.cardSize() / (1024 * 1024);

Serial.printf("SD Card Size: %llu MB\n", cardSize);

// List files on SD card

Serial.println("Files on SD:");

listDir(SD, "/", 0);

}

// ---- Audio init ----

out = new AudioOutputI2S(0, AudioOutputI2S::INTERNAL_DAC);

out->SetOutputModeMono(true);

out->SetGain(0.8);  // volume

}

/***************  LOOP  ***************/

void loop() {

// Clap detection

int clapVal = digitalRead(SOUND_PIN);

if (clapVal == LOW) {

Serial.println("Clap detected!");

digitalWrite(BUZZER_PIN, HIGH);

delay(200);

digitalWrite(BUZZER_PIN, LOW);

// Display welcome

display.clearDisplay();

display.setTextSize(2);

display.setCursor(15, 20);

display.println("Welcome!");

display.display();

delay(2000);

// Show sensor readings

display.clearDisplay();

display.setTextSize(1);

display.setCursor(0,0);

// Temperature

float temp = dht.readTemperature();

display.print("Temp: ");

display.print(temp);

display.println(" C");

// Pressure

sensors_event_t event;

bmp.getEvent(&event);

if (event.pressure) {

display.print("Pressure: ");

display.print(event.pressure);

display.println(" hPa");

}

// Air Quality from MQ135

int airVal = adc1_get_raw(ADC1_CHANNEL_6);

display.print("Air: ");

display.println(airVal);

if (airVal > 200) {

display.println("BAD WEATHER");

}

display.display();

delay(3000);

// Back to waiting screen

display.clearDisplay();

display.setTextSize(2);

display.setCursor(10, 20);

display.println("EchoMirror");

display.setTextSize(1);

display.setCursor(20, 45);

display.println("Waiting for clap...");

display.display();

}

}